# Technical Engineering and Execution Procedure

Project Team Name: SpyKids						Document No: SK-P1-003

Mobile Application: Baobei Parent Hub

Bear: Baobei Child Smart Bear

Purpose: Working engineering reference for the Milestone 1 (Week 8) interactive core build, extending the system architecture and design solution described in the Assessment 1 proposal (“Bridging the Emotional Disconnect: Designing an Interactive Digital Parental Presence for Left-Behind Children”). This document takes the block-level architecture already agreed and turns it into a buildable wiring and assembly procedure, with the safety controls the build needs.

Status: Draft for team review — component choices and pin assignments should be confirmed against the physical dev boards purchased before soldering begins.

# 1. Scope and System Recap

The bear's electronics implement the “Embodied Companion Touchpoint” described in the proposal: squeeze-activated playback, squeeze-activated recording, an ambient LED heartbeat, and offline-tolerant local caching. This procedure covers the physical build only — power, core, audio, sensing, and connectivity subsystems — not the firmware logic or the mobile app / cloud broker, which are addressed separately.

Five subsystems, as previously scoped:

| Subsystem | Core components |
| --- | --- |
| Power | Li-po cell, magnetic charging dock, TP4056 charge/protection module, 5V boost, 3.3V logic rail |
| Core | ESP32-WROOM-32 dev board |
| Audio I/O | INMP441 I2S MEMS microphone, MAX98357A I2S class-D amplifier, 4Ω+ speaker |
| Sensors & LED | Two paw squeeze switches, one WS2812 (NeoPixel) LED |
| Connectivity & storage | Onboard Wi-Fi, microSD (or DFPlayer Mini as an audio-storage fallback) |

Open scope item: the individual proposal includes an eye-camera for video replies. This build procedure does not include a camera subsystem — the team's earlier direction was audio-only, for bandwidth, data-safety, and stigma reasons. Confirm with the team before sourcing camera hardware; adding one later is a distinct hardware and firmware task, not a drop-in addition to this wiring plan.

# 2. Bill of Materials and Recommended Pin Map

| Ref | Component | Notes |
| --- | --- | --- |
| U1 | ESP32-WROOM-32 dev board | 34 usable GPIO; GPIO6–11 reserved for onboard SPI flash and must not be reused (Espressif Systems, n.d.-a) |
| U2 | INMP441 I2S MEMS microphone breakout | 1.8–3.3V logic, I2S digital output |
| U3 | MAX98357A I2S class-D amplifier breakout | 2.5–5.5V VIN, drives 4Ω+ speaker directly (Adafruit Industries, n.d.-a) |
| SPK1 | Small speaker, ≥4Ω | Must not be lower impedance than 4Ω — the MAX98357A is only rated to drive 4Ω or higher (Adafruit Industries, n.d.-a) |
| LED1 | WS2812 / NeoPixel, single pixel | Chest “heart” indicator |
| SW1, SW2 | Tactile push switches | Left paw (playback), right paw (record) |
| BAT1 | Li-po cell, 3.7V nominal, 1200–2000mAh | Size to fit the plush cavity |
| U4 | TP4056-based charge/protection module | Feeds from the magnetic dock |
| U5 | 5V boost converter | Powers amp + LED rail |
| Optional | DFPlayer Mini + microSD | Local fallback playback / offline cache |

Recommended pin assignments (confirm against your specific dev board silkscreen before soldering — pin numbering varies slightly by board revision):

| Signal | GPIO | Notes |
| --- | --- | --- |
| I2S0 – Mic SCK (bit clock) | GPIO14 |  |
| I2S0 – Mic WS (word select) | GPIO15 |  |
| I2S0 – Mic SD (data out) | GPIO32 |  |
| Mic L/R select | GND | Selects left channel (Robogears, n.d.) |
| I2S1 – Amp BCLK | GPIO26 | Second, independent I2S peripheral so mic input and speaker output can run concurrently |
| I2S1 – Amp LRC | GPIO25 |  |
| I2S1 – Amp DIN | GPIO22 |  |
| NeoPixel data | GPIO4 | Through a 330–470Ω series resistor at the LED end (Adafruit Industries, n.d.-b) |
| Left paw switch | GPIO13 | Internal pull-up, switch to GND |
| Right paw switch | GPIO27 | Internal pull-up, switch to GND |
| DFPlayer UART TX/RX | GPIO17 / GPIO16 | Optional fallback path |

GPIO6–11 are wired internally to the module's SPI flash and are not available for general use (Espressif Systems, n.d.-b). GPIO34–39 are input-only and cannot drive outputs such as the LED or amplifier (Electronify India, n.d.) — avoid assigning outputs to this range.

# 3. Pre-Build Safety Briefing

Read this section fully before any soldering or battery handling. Several of these hazards are specific to a battery-powered plush toy intended for a child aged 6–12, not just to the electronics bench.

## 3.1 Soldering Hazards

- Iron tips exceed 400°C and cause instant burns on contact with skin (Columbia University Research, n.d.). Never touch the tip or a freshly soldered joint — let joints cool before handling.

- Solder fumes (rosin/flux smoke) irritate eyes and airways; keep your face out of the smoke path and work in a ventilated area or with a fume extractor (Duke Pratt ECE Lab, n.d.; RPI EHS, n.d.).

- Wear ANSI-rated safety glasses — solder can spit (University of Illinois DRS, n.d.).

- Use a proper stand for the iron; never rest it on the bench, and unplug it whenever you step away (BE Labs, University of Pennsylvania, n.d.).

- If burned: cold running water for 15 minutes, no creams, cover with a clean dressing (Columbia University Research, n.d.).

## 3.2 Battery (Li-po) Hazards

- Never exceed 4.2V per cell when charging; overcharging can cause overheating, swelling, or fire (Hubei Liju New Energy, 2025; MaxAmps, n.d.). Only use a charge circuit designed for Li-po (the TP4056 module specified above).

- Never leave a Li-po charging unattended, and charge on a fireproof surface, away from flammable materials (Ryan Hobbies, 2025; ODG, 2026).

- Never puncture, crush, or short the cell's leads together — if you must cut leads, cut them one at a time so the exposed ends can't touch (Ryan Hobbies, 2025).

- Never charge or use a swollen/ballooned cell (Ryan Hobbies, 2025).

- Because this product is intended for a child, the battery and all charge circuitry must sit inside a compartment the child cannot access during normal play — charging should happen only via the sealed magnetic dock, not through an openable panel.

## 3.3 Toy-Safety and Child-Product Obligations

Because the artefact is a battery-powered electronic toy for a 6–12 year-old, the physical build should be evaluated against the small-parts and battery-containment expectations in the relevant toy-safety frameworks, even at prototype stage:

- Small internal components (switches, connectors, the LED, loose wiring) must not be reachable or removable by the child — this is the core small-parts/choking concern in toy safety standards for this age range (ASTM International, 2023; LISUN, n.d.).

- Battery compartments in commercial electric toys are increasingly required to resist a child prying them open — even at prototype stage, fasten your battery bay with real screws, not just tape or velcro (Federal Register, 2024).

- These are prototype-stage design disciplines, not a compliance claim — a Milestone 1/2 prototype does not need to pass formal certification, but building to these habits now avoids rework later and protects whoever handles it during testing.

## 3.4 Electrical and Moisture Risks

- Never solder on a live/powered circuit.

- Inspect the iron's cord before each session for fraying or damage (RPI EHS, n.d.).

- Because this is a soft-toy enclosure, condensation and handling moisture (little hands, drool, spilled drinks) are a real risk to exposed PCBs. Breakout boards should be conformal-coated or fully enclosed in a hard inner shell before the plush shell goes over them — MEMS sensors themselves are typically sealed, but exposed traces and headers are not (Zbotic, 2026).

# 4. Tools and Workspace Setup

- Temperature-controlled soldering iron (lead-free solder: ~320–350°C tip temperature), stand, brass/wet sponge

- Fume extractor or well-ventilated bench

- Fine-tip tweezers, flush cutters, wire strippers

- Multimeter (continuity + voltage)

- Helping-hands or PCB vice

- Heat-shrink tubing + heat gun/lighter

- ANSI-rated safety glasses

- A fire-safe surface (ceramic tile) and a Li-po charging bag for all charge cycles

- Anti-static mat/wrist strap (recommended, not mandatory for this parts list)

# 5. Step-by-Step Wiring and Assembly Procedure

Work in this order — each stage is tested with the multimeter before power is applied, and each subsystem is bench-tested individually before final assembly into the plush shell.

## Step 1 — Power Subsystem

- 1. Solder the Li-po's JST connector leads to the TP4056 module's B+/B- pads (check polarity twice — reversed leads are a short-circuit risk).

- 2. Wire the magnetic pogo-pin dock's two contacts to the TP4056's IN+/IN- pads.

- 3. Wire the TP4056's protected output (OUT+/OUT-) to the 5V boost converter input.

- 4. Bench-test this stage alone first: connect the Li-po, place it in a Li-po-safe bag, and confirm on the multimeter that the boost converter outputs a steady 5V before connecting anything downstream.

- 5. Tap the boost converter's 5V rail to power the ESP32 dev board's VIN/5V pin, the MAX98357A VIN, and the NeoPixel VCC. The ESP32 board's onboard regulator will supply 3.3V internally for logic and the INMP441.

## Step 2 — ESP32 Core Bring-Up

- 1. Before wiring anything else, flash a minimal blink/serial-print sketch to the bare ESP32 board over USB and confirm it boots — this isolates core failures from wiring failures later.

- 2. Mark GPIO6–11 as off-limits on your wiring notes (SPI flash) and GPIO34–39 as input-only (Espressif Systems, n.d.-a; Electronify India, n.d.).

## Step 3 — Audio Output (MAX98357A + Speaker)

- 1. Solder BCLK, LRC, DIN, GND, VIN header pins onto the MAX98357A breakout if not pre-populated.

- 2. Wire BCLK→GPIO26, LRC→GPIO25, DIN→GPIO22, GND→common ground, VIN→5V rail.

- 3. Leave the GAIN pin unconnected for the default 9dB gain, or set it per the datasheet if a louder/quieter output is needed (Adafruit Industries, n.d.-a).

- 4. Solder the speaker leads to the amplifier's +/- output terminals. Do not connect this output to any other amplifier stage — it drives the speaker directly (Adafruit Industries, n.d.-a).

- 5. Flash a basic I2S tone-generation sketch and confirm audio output before moving on.

## Step 4 — Audio Input (INMP441 Microphone)

- 1. Wire VDD→3.3V, GND→common ground, L/R→GND (selects left channel) (Robogears, n.d.).

- 2. Wire SCK→GPIO14, WS→GPIO15, SD→GPIO32.

- 3. Flash a basic I2S read-and-print sketch, speak near the mic, and confirm non-zero sample values in serial output.

## Step 5 — Sensors (Paw Switches)

- 1. Wire one leg of each tactile switch to GND, the other to its GPIO (GPIO13 left, GPIO27 right), using the ESP32's internal pull-up resistors in firmware — no external resistor needed.

- 2. Test each switch with a simple digital-read print sketch before embedding it in the paw.

## Step 6 — LED (NeoPixel Heart)

- 1. Solder a 330–470Ω resistor in series on the data line, placed as close to the LED as physically possible (Adafruit Industries, n.d.-b; ComponentIndex, 2026).

- 2. Wire VCC→5V rail, GND→common ground, DIN→GPIO4 (through the resistor).

- 3. If you later chain more than one pixel, add a 1000µF capacitor across the 5V/GND rail at the LED end to absorb inrush current (Adafruit Industries, n.d.-b).

- 4. Run the Adafruit NeoPixel library's basic colour-cycle example to confirm.

## Step 7 — Connectivity & Storage (Optional DFPlayer Path)

- 1. If using the DFPlayer Mini as a local fallback/offline cache, wire its RX→GPIO17, TX→GPIO16 (through a voltage divider if the DFPlayer's RX is not 3.3V-tolerant — check your specific module), VCC→5V, GND→common ground.

- 2. Insert a formatted microSD card loaded with test audio clips and confirm playback via serial command before integration.

## Step 8 — Full Bench Integration Test

- 1. With all subsystems wired to a shared ground and the shared 5V/3.3V rails, power up from the Li-po only (not USB) and confirm: paw switch press triggers audio playback from the last cached message; held paw switch triggers a recording via the mic, saved locally; the NeoPixel pulses on new-message state.

- 2. Measure current draw at idle and at peak (playback + LED) with the multimeter to sanity-check your battery life estimate.

- 3. Only once this bench test passes, proceed to enclosure integration.

## Step 9 — Enclosure Integration into the Plush Shell

- 1. Conformal-coat or box each bare breakout board before it goes near fabric or stuffing (Zbotic, 2026).

- 2. Mount the battery and charge module in a screw-fastened internal compartment the child cannot open by hand (Federal Register, 2024).

- 3. Route the magnetic dock contacts to the rear exterior seam only — no other wiring should be exposed at the plush surface.

- 4. Secure all connectors with hot glue or heat-shrink strain relief before final stuffing — a loose header pin becomes a small-parts hazard once sewn inside a toy (ASTM International, 2023).

- 5. Re-run the Step 8 integration test after the shell is closed, since antenna performance (Wi-Fi) and speaker resonance can both change once the electronics are inside fabric and stuffing.

Insert Smart Bear Acrhitecture here

# 6. Risk Register

| Risk | Likelihood | Severity | Mitigation |
| --- | --- | --- | --- |
| Li-po overcharge / fire | Low, if procedure followed | High | TP4056 protected charge module, charge only in Li-po bag, never unattended (MaxAmps, n.d.; Ryan Hobbies, 2025) |
| Solder burns | Medium | Medium | Iron stand discipline, cool-down time, first-aid steps documented above |
| Flux fume exposure | Medium | Low–Medium | Ventilated bench / fume extractor, avoid leaning over the joint (RPI EHS, n.d.) |
| Loose small parts (switches, header pins, LED) becoming a choking hazard | Medium | High (child product) | Heat-shrink/hot-glue strain relief, screw-fastened compartments, no exposed loose hardware in final shell (ASTM International, 2023) |
| Battery compartment opened by child | Low–Medium | High | Screw-fastened internal bay, charging only via sealed magnetic dock, no user-openable panel (Federal Register, 2024) |
| Moisture ingress (handling, drool, spills) | Medium | Medium | Conformal coating / hard inner enclosure for all boards (Zbotic, 2026) |
| Reversed battery polarity during assembly | Low | High | Multimeter check before every power-on, double-check JST polarity before soldering |
| GPIO conflict with SPI flash pins (bricking the board) | Low, if pin map followed | Medium | Avoid GPIO6–11 entirely; confirm against board silkscreen before wiring (Espressif Systems, n.d.-a) |
| Wi-Fi timing interference with NeoPixel data line | Low | Low | Use the RMT or I2S driver for LED output rather than bit-banging (ComponentIndex, 2026) |
| Unauthenticated pairing / Wi-Fi credential exposure — a stranger's app claims or eavesdrops on a bear over shared Wi-Fi | Low | High (privacy) | Per-device pairing token generated at first pair (not a fixed default), cloud security rules scoped to the matched bear_id/parent_account_id pair (Espressif Systems, n.d.-c) |

# 7. Costed Bill of Materials (AUD)

Pricing below is per single bear unit, checked against Australian retailers at time of writing. Prices marked “(estimate)” are typical market rates rather than a specific live listing — either because the item isn't a standard local retail line (e.g. the magnetic dock connector) or the retailer's listing didn't expose a clean unit price. GST is included where the retailer states so (Core Electronics confirms all listed prices are GST-inclusive). Shipping is not included; a single consolidated order from one retailer typically adds $7–11 AUD.

## 7.1 Electronics

| Component | Price (AUD) | Source |
| --- | --- | --- |
| FireBeetle ESP32 dev board (DFR0478) | $18.70 | https://core-electronics.com.au/firebeetle-esp32-iot-microcontroller-supports-wi-fi-bluetooth.html |
| INMP441 I2S MEMS microphone (CE10259) | $12.30 | https://core-electronics.com.au/i2s-microphone-inmp441.html |
| MAX98357 I2S amplifier module (DFR0954) | $10.15 | https://core-electronics.com.au/max98357-i2s-amplifier-module.html |
| Li-po 3.7V 1100mAh battery (CE04377) | $8.65 | https://core-electronics.com.au/polymer-lithium-ion-battery-1000mah-38458.html |
| TP4056 Li-po charger module (DFR0667) | $9.45 | https://core-electronics.com.au/lipo-charger-microusb.html |
| DFPlayer Mini (DFR0299, optional fallback path) | $8.55 | https://core-electronics.com.au/dfplayer-a-mini-mp3-player.html |
| 4Ω+ speaker, small | $8.95 | https://core-electronics.com.au/mono-enclosed-speaker-3w-4-ohm.html |
| 5V boost converter | ~$7.00 (estimate) | https://core-electronics.com.au/dc-dc-usb-0-9v-5v-to-5v-dc-boost-step-up-power-supply-module.html |
| microSD card, 16GB | ~$12.00 (estimate) | https://core-electronics.com.au/sd-microsd-memory-card-16gb-class-10.html |
| Magnetic pogo-pin charging dock (2-pin) | ~$8.00 (estimate) | ??? |
| NeoPixel, single pixel | ~$3.50 (estimate) | https://core-electronics.com.au/ultra-bright-3-watt-chainable-neopixel-led-ws2811.html |
| Paw tactile switches × 2 | ~$2.00 (estimate) | https://core-electronics.com.au/soft-tactile-button-8mm-x-10.html |
| Wire, resistors, heat-shrink, hot glue allowance | ~$10.00 (estimate) | https://core-electronics.com.au/added/?product=419 |
| Electronics subtotal | ≈ $118.44 |  |

## 7.2 Physical Shell Materials

| Material | Price (AUD) | Source |
| --- | --- | --- |
| Faux fur fabric, short/medium pile, ~0.5m | ~$12–15 (current Lincraft sale rate is $12/m; regular price is typically higher) | Lincraft |
| Polyester fibre toy stuffing, 500g | ~$10.00 (estimate) | Lincraft |
| PLA filament for internal frame/enclosure, ~150g of a 1kg spool | ~$5.50 (Polymaker PolyLite PLA 1KG is $36.95; only a fraction used per bear) | Core Electronics |
| Felt, thread, safety eyes, trim allowance | ~$8.00 (estimate) | — |
| Fasteners/screws for the battery compartment | ~$5.00 (estimate) | — |
| Shell materials subtotal | ≈ $40.50 |  |

## 7.3 Grand Total

≈ $159 AUD per bear, before shipping. This is a per-unit figure — multiply by however many bears the team builds across Milestone 1 and Milestone 2 (a full rebuild for Milestone 2 is not necessarily required if the Milestone 1 electronics core is reused inside a refined shell).

A couple of things worth double-checking before committing this to the shared repo:

- The FireBeetle ESP32 board has an onboard Li-po charging circuit — worth confirming whether it makes the separate TP4056 module redundant, which would shave about $8.59 off the electronics subtotal.

- The magnetic dock connector price is the least certain line item here; get an actual quote from an AliExpress listing before finalising the budget, since pricing varies a lot by minimum order quantity.

# 8. Device Pairing and App Connectivity

Status: newly identified gap — not yet confirmed with the team. The connectivity subsystem as originally scoped (“onsite Wi-Fi, microSD”) describes the network layer but never specifies how a specific bear gets associated with a specific family's app, or how that link is secured. This section closes that gap. Flag it to Charles/CJ/Yajie/Anton/Simon before it's treated as locked, since it introduces a security model the team hasn't reviewed.

## 8.1 Why a Separate Pairing Layer Is Needed

A teddy bear has no keyboard or screen, so it can't be handed Wi-Fi credentials directly. The parent's app also needs a reliable way to know which physical bear it's talking to — there could be more than one in a household, or a neighbour's bear could be in Bluetooth range. Both problems are solved by a short-range pairing step that happens once, before the bear ever talks to the cloud.

## 8.2 Two-Layer Connection Model

No new hardware is required — the ESP32 already carries both Wi-Fi and Bluetooth Low Energy (BLE) radios on the same chip (Espressif Systems, n.d.-a).

Pairing/setup layer — BLE. Used only during first-time setup (and again after a factory reset). The bear advertises itself; the app connects locally over Bluetooth to hand over Wi-Fi credentials and a pairing token. No internet connection is needed for this step.

Operational layer — Wi-Fi + cloud broker. Once paired, all ongoing voice-message sync happens exactly as described in Section 1 and the system architecture — Wi-Fi to the cloud broker, store-and-forward when offline. BLE can idle to save power, or stay available as a local fallback channel for diagnostics (battery %, last sync time, firmware version) when Wi-Fi is down.

## 8.3 Pairing Sequence

- 1. App: open pairing mode. Parent taps "Add bear" in the app.

- 2. Bear: BLE advertising. The bear broadcasts a unique bear ID over BLE.

- 3. App: send Wi-Fi + token. The app connects over BLE and writes the home Wi-Fi SSID, password, and a freshly generated pairing token to the bear.

- 4. Bear: join Wi-Fi and register. The bear saves the credentials to flash (NVS), connects to Wi-Fi, and calls the cloud broker with its bear ID and pairing token.

- 5. Cloud broker: confirm pairing. The broker links bear_id to parent_account_id and confirms back to both devices.

- 6. App: bear connected. Ongoing sync now runs over Wi-Fi and the cloud broker, as already documented.

This is the standard pattern implemented by Espressif's wifi_provisioning component in ESP-IDF (Espressif Systems, n.d.-c) — it doesn't need to be built from scratch, which keeps this addition low-risk from an engineering-effort standpoint even though it's new scope from a project-management standpoint.

## 8.4 Security Considerations

- The pairing token must be generated fresh per device at first pair, not shipped as a fixed default — a hardcoded token would let any app claim any bear.

- Cloud-side security rules must scope every message and status read to the matched bear_id/parent_account_id pair, so a bear can't be read or written to by an account it isn't paired with (Espressif Systems, n.d.-c).

- This closes the "unauthenticated pairing" risk added to the register in Section 6.

# 9. Mobile App Architecture

Status: proposed stack — not yet confirmed with the team. This section mirrors the depth of Section 8, but for the app side of the same pairing/sync relationship. It proposes a concrete framework and library stack rather than leaving “the app” as an unspecified box in the system diagram — flag the stack choice to the team, since swapping frameworks later is expensive.

Insert Mobile App Acrhitecture here

## 9.1 Recommended Stack

| Layer | Choice | Why |
| --- | --- | --- |
| Framework | Flutter (Dart) | Single codebase for iOS and Android, strong hot-reload workflow suited to an iterative studio timeline (Flutter, n.d.) |
| BLE | flutter_blue_plus | Actively maintained plugin for scanning, connecting, and reading/writing BLE GATT characteristics — drives the Section 8 pairing flow (pub.dev, n.d.-a) |
| Auth | firebase_auth | Parent account sign-in |
| Message data + sync | cloud_firestore | Cloud-hosted NoSQL database with live sync and built-in offline persistence (pub.dev, n.d.-b) |
| Audio storage | firebase_storage | Object storage for voice-message audio files (pub.dev, n.d.-c) |
| Push notifications | firebase_messaging | Notifies the parent when a new message arrives from the bear (pub.dev, n.d.-d) |
| Secure local storage | flutter_secure_storage | Stores the pairing token in the OS keychain/keystore rather than plain preferences storage (pub.dev, n.d.-e) |

Alternative: React Native is a reasonable substitute if the team has more JavaScript than Dart experience — the four-layer architecture below still applies, swapping flutter_blue_plus for react-native-ble-plx and the FlutterFire plugins for the React Native Firebase SDK. The choice should be made on team comfort level, not a perceived “better” option — both are production-viable for this scope.

## 9.2 Software Architecture (Four Layers)

This matches the block diagram above:

- Connectivity layer. Talks to the bear over BLE during pairing only, and to the cloud broker over the Firebase SDKs for everything after that. This is the only layer that knows about either external system — everything else in the app talks to it, not directly to the bear or the cloud.

- State & sync manager. Holds app-wide state (current bear, pairing status, message list), manages the offline send queue, and triggers local push-notification handling. This is the layer that decides what the UI should currently show.

- Presentation layer. Screens and UI state only — no direct network or BLE calls.

- Local storage. Two distinct things live here and should not be conflated: a message cache (for offline viewing, non-sensitive) and a secure token store (the pairing token, sensitive — see Section 9.5).

## 9.3 Screen Map

| Screen | Purpose |
| --- | --- |
| Onboarding & bear pairing | Runs the BLE pairing sequence from Section 8.3 |
| Home / message timeline | List of sent and received voice messages, newest first |
| Record & send | Tap-to-record voice message interface with playback preview before sending |
| Bear status | Battery level, last-sync time, connection health |
| Settings | Manage multiple bears, notification preferences, account |

## 9.4 Offline and Sync Behaviour

Cloud Firestore's built-in offline persistence caches recently-synced documents locally, so the message timeline still renders without a live connection; outgoing messages queue locally and flush automatically once connectivity returns (pub.dev, n.d.-b). This mirrors the store-and-forward behaviour already designed for the bear side (Section 1, cloud sync broker) — the offline-tolerance story is now consistent end to end, not just on the hardware side.

## 9.5 Security

- The pairing token is written to flutter_secure_storage (OS keychain on iOS, encrypted keystore on Android), never to plain preferences storage (pub.dev, n.d.-e).

- Parent accounts authenticate through Firebase Auth; Firestore and Storage security rules are scoped per parent_account_id, matching the access-control model introduced in Section 8.4 — this section and Section 8 describe the two matching halves of the same security boundary.

# 10. References

Core Electronics. (n.d.-a). FireBeetle ESP32 IOT microcontroller (supports Wi-Fi & Bluetooth). https://core-electronics.com.au/firebeetle-esp32-iot-microcontroller-supports-wi-fi-bluetooth.html

Core Electronics. (n.d.-b). I2S microphone (INMP441). https://core-electronics.com.au/i2s-microphone-inmp441.html

Core Electronics. (n.d.-c). MAX98357 I2S amplifier module. https://core-electronics.com.au/max98357-i2s-amplifier-module.html

Core Electronics. (n.d.-d). Polymer lithium ion battery (LiPo) 3.7V 1100mAh. https://core-electronics.com.au/polymer-lithium-ion-battery-1000mah-38458.html

Core Electronics. (n.d.-e). Lipo charger-microUSB (TP4056-based). https://core-electronics.com.au/lipo-charger-microusb.html

Core Electronics. (n.d.-f). DFPlayer - a mini MP3 player. https://core-electronics.com.au/dfplayer-a-mini-mp3-player.html

Core Electronics. (n.d.-g). Polymaker PolyLite black PLA filament 1KG 1.75mm. https://core-electronics.com.au/3d-printers-australia/filaments.html

Lincraft. (n.d.-a). Faux fur fabric, width 75cm. https://lincraft.com.au/products/faux-fur-30308671

Lincraft. (n.d.-b). Makr fairy floss polyester fill. https://lincraft.com.au/products/fairy-floss-poly-fill-siliconisd-30342541

Adafruit Industries. (n.d.-a). Adafruit MAX98357 I2S class-D mono amp. Adafruit Learning System. https://learn.adafruit.com/adafruit-max98357-i2s-class-d-mono-amp/overview

Adafruit Industries. (n.d.-b). Adafruit NeoPixel Überguide: Powering NeoPixels. Adafruit Learning System. https://learn.adafruit.com/adafruit-neopixel-uberguide/powering-neopixels

ASTM International. (2023). ASTM F963-23: Standard consumer safety specification for toy safety [Summary]. The ANSI Blog. https://blog.ansi.org/ansi/astm-f963-23-standard-consumer-toy-safety/

BE Labs, University of Pennsylvania. (n.d.). Soldering safety and procedures. https://belabs.seas.upenn.edu/resources-tutorials/soldering-safety-and-procedures

Columbia University Research. (n.d.). Safely handling solder. https://research.columbia.edu/ehs/safely-handling-solder

ComponentIndex. (2026, March 11). WS2812B NeoPixel guide: Wiring, code & pinout for Arduino, ESP32 & Pi Pico. https://componentindex.net/components/ws2812b/

Duke Pratt School of Engineering, ECE Lab. (n.d.). Safety: Soldering rules. https://ecelab.pratt.duke.edu/safety

Electronify India. (n.d.). ESP32-WROOM-32 datasheet explained: Specifications, pinout & performance. https://www.electronifyindia.com/blogs/news/esp32-wroom-32-datasheet

Espressif Systems. (n.d.-a). ESP32-WROOM-32 datasheet (v3.7). https://documentation.espressif.com/esp32-wroom-32_datasheet_en.html

Espressif Systems. (n.d.-b). ESP32-WROOM-32 datasheet (v2.7). https://cdn.sparkfun.com/assets/learn_tutorials/8/5/2/esp32-wroom-32_datasheet_en.pdf

Espressif Systems. (n.d.-c). Wi-Fi provisioning — ESP-IDF programming guide. https://docs.espressif.com/projects/esp-idf/en/latest/esp32/api-reference/provisioning/wifi_provisioning.html

Flutter. (n.d.). Flutter documentation. https://docs.flutter.dev/

pub.dev. (n.d.-a). flutter_blue_plus. https://pub.dev/packages/flutter_blue_plus

pub.dev. (n.d.-b). cloud_firestore. https://pub.dev/packages/cloud_firestore

pub.dev. (n.d.-c). firebase_storage. https://pub.dev/packages/firebase_storage

pub.dev. (n.d.-d). firebase_messaging. https://pub.dev/packages/firebase_messaging

pub.dev. (n.d.-e). flutter_secure_storage. https://pub.dev/packages/flutter_secure_storage

Federal Register. (2024, August 13). Safety standard for toys: Requirements for toys containing button cell or coin cell batteries [Proposed rule, U.S. Consumer Product Safety Commission]. https://www.federalregister.gov/documents/2024/08/13/2024-17472/safety-standard-for-toys-requirements-for-toys-containing-button-cell-or-coin-cell-batteries

Hubei Liju New Energy Co., Ltd. (2025, September 20). Lithium polymer battery safety tips for consumers. https://hbljdc.com/news/lithium-polymer-battery-safety-tips-for-consumers.html

LISUN. (n.d.). ASTM F963:2023 standard consumer safety specification for toy safety. https://www.lisungroup.com/standards/astm-f9632023-standard-consumer-safety-specification-for-toy-safety.html

MaxAmps. (n.d.). Lithium polymer charging/discharging & safety information. https://maxamps.com/pages/lipo-care-php

ODG. (2026, May 27). A beginner's guide to LiPo battery charging and storage. https://www.origin-ic.com/blog/guide-safe-lithium-polymer-battery-charging-storage/48438

Robogears. (n.d.). INMP441 MEMS omnidirectional microphone module: ESP32 connection guide. https://robogearsin.odoo.com/hi/shop/22558877-inmp441-mems-omnidirectional-microphone-module-high-precision-snr-low-power-i2c-interface-supports-esp32-20

RPI Environmental Health & Safety. (n.d.). Soldering safety guidelines. https://ehs.rpi.edu/lab-research-shop-safety/soldering-safety-guidelines

Ryan Hobbies. (2025, June 25). Lithium battery safety! https://ryanhobbies.com/lithium-battery-safety/

University of Illinois, Division of Research Safety. (n.d.). Soldering safety. https://drs.illinois.edu/Page/SafetyLibrary/SolderingSafety

Zbotic. (2026, March 11). INMP441 microphone ESP32: Voice recognition project guide. https://zbotic.in/inmp441-microphone-esp32-voice-recognition-project-guide

This document is a living reference — update the pin map and BOM if component substitutions are made during procurement, and re-confirm the risk register once the physical shell design is finalised.